ITA1447 - ETHICAL HACKING FOR ENUMERATING WINDOWS

LAB MANUAL

Experiment No: 3

Cracking the Password using Hydra

Aim:

To crack FTP service credentials of a target machine using the Hydra password-cracking tool.

Procedure:

- Open Kali Linux and navigate to Applications -> Password Attacks -> Online Attacks -> hydra, or open a terminal and type hydra.

- Identify the target service to attack; in this case, the FTP service running on the Metasploitable machine with IP address 192.168.1.101.

- Prepare a wordlist containing possible usernames and passwords. Create (or use an existing) wordlist file with the extension .lst, stored at the path usr/share/wordlist/Metasploit.

- Open a terminal window on Kali Linux.

- Construct the Hydra command specifying the login wordlist file (-l) and the password wordlist file (-P), along with the target service and IP address.

- Add the -V flag to display each username and password combination being attempted in real time.

- Execute the Hydra command and allow it to attempt each combination against the FTP service.

- Observe the output as Hydra reports each attempt, including successful and failed logins.

- Record the valid username and password combination once found (msfadmin:msfadmin in this case).

- Verify the discovered credentials by manually logging into the FTP service using an FTP client.

Command

hydra -l /usr/share/wordlists/metasploit/user -P \ /usr/share/wordlists/metasploit/password ftp://192.168.1.101 -V

Output:

Result:

Hydra was used to successfully brute force the FTP login credentials of the target machine, and the valid credentials (msfadmin:msfadmin) were obtained.